FIRST SCHEDULE

[Paragraphs 2 (5) and 4 (6)]

Part A – TABLE OF GENERAL CLASSIFICATION OF AIRCRAFT

Aircraft (Power-driven flying machines)Aeroplane (Landplane)Aeroplane (Seaplane)Aeroplane (Amphibian)Helicopter (Landplane)Helicopter (Seaplane)

Part B – ORIGIN AND REGISTRATION MARKS OF AIRCRAFT REGISTERED IN MACAO

1.	The mark of origin of the aircraft registered in Macao shall be the roman capital letter "B", and the registration mark shall be a group of 3 capital letters in roman characters assigned by the Civil Aviation Authority on the registration of the aircraft. A hyphen shall be placed between the mark of origin and the registration mark.

2.	The origin and registration marks shall be painted on the aircraft or shall be affixed thereto by any other means ensuring a similar degree of permanence in the following manner:

(1)	Position of marks.

(a)	All aircraft

(i)	Wings – On aeroplanes, the marks shall appear once on the lower surface of the wing structure. They shall be located on the left half of the lower surface of the wing structure unless they extend across the whole of the lower surface of the wing structure. So far as possible the marks shall be located equidistant from the leading and trailing edges of the wings. The tops of the letters and numbers shall be towards the leading edge of the wing.

(ii)	Fuselage (or equivalent structure) and vertical tail surface – On all aircraft the marks shall also be either on each side of the fuselage (or equivalent structure) between the wings and the tail surfaces, or on the upper halves of the vertical tail surfaces. When located on a single vertical tail surface they shall be on both sides of the tail. When there is more than one vertical tail surface, the marks shall appear on the outboard sides of the outer surface.

(iii)	If an aircraft does not possess parts corresponding to those mentioned in (i) and (ii) above the marks shall appear in a manner such that the aircraft can be identified readily.

(2)	Size of marks.

(a)	All aircraft

(i)	Wings – The marks on the wings shall be of equal height and at least 50 cm in height.

(ii)	Fuselage (or equivalent structure) or vertical tail surface – The marks on the fuselage (or equivalent structure) shall not interfere with the visible outlines of the fuselage (or equivalent structure). The marks on the vertical tail surfaces shall be such as to leave a margin of at least 5 cm along each side of the vertical tail surface. The letters and numerals constituting each group of marks shall be of equal height. The height of the marks shall be at least 30 cm: Provided that where owing to the structure of the aircraft a height of 30 cm is not reasonably practicable the height shall be the greatest height reasonably practicable in the circumstances, but not less than 15 cm.

(iii)	Special cases – If an aircraft does not possess parts corresponding to those mentioned in sub-paragraphs (i) and (ii) above, the measurements of the marks shall be such that the aircraft can be identified readily.

(3)	Form, width and spacing of marks.

(i)	The letters shall be capital letters in roman characters without ornamentation. Numbers shall be Arabic numbers without ornamentation.

(ii)	The width of each character (except the letter I and the number 1) and the length of hyphens shall be two-thirds of the height of a character.

(iii)	The characters and hyphens shall be formed by solid lines and shall be of a colour contrasting clearly with the background. The thickness of the lines shall be one-sixth of the height of a character.

(iv)	Each character shall be separated from that which it immediately precedes or follows by a space equal to half the width of a character. A hyphen shall be regarded as a character for this purpose.

3.	The origin and registration marks shall be displayed to the best advantage, taking into consideration the constructional features of the aircraft and shall always be kept clean and visible.

4.	In addition to paragraphs 1 to 3, the origin and registration marks shall also be inscribed, together with the name and address of the registered owner of the aircraft, on a fire-proof metal plate affixed in a prominent position to the aircraft near the main entrance.